# Pantry Shelf Rebuild

Workshop project plan and cut list - C. Whitfield

# Overview

Replacing the three sagging pine shelves in the kitchen pantry with a stronger ladder-style unit in poplar. Carcass joined with pocket screws, shelves on adjustable pins so the spacing can change later. Finish to match the trim, a wipe-on satin poly. Target two Saturdays in the shop, glue-up the first weekend, finish the second.

# Materials

| Item | Spec | Qty | Source |
| --- | --- | --- | --- |
| Poplar board | 3/4 in x 11-1/4 in x 8 ft | 4 | Specialty wood supplier (south of Portland) |
| Poplar board | 3/4 in x 2-1/2 in x 8 ft (face frame) | 2 | Same |
| Pocket screws | 1-1/4 in coarse | 1 box | Home Depot |
| Shelf pins | 5 mm, nickel | 1 pack | Home Depot |
| Wipe-on poly | satin, quart | 1 | Home Depot |
| Sandpaper | 120 / 240 / 320 grit | 1 pack each | On hand |

# Cut List

| Part | Qty | Finished size (in) | Material | Notes |
| --- | --- | --- | --- | --- |
| Side panel | 2 | 11-1/4 x 64 | Poplar 3/4 | Drill pin holes at 1-1/4 spacing |
| Fixed top | 1 | 11-1/4 x 28-1/2 | Poplar 3/4 | Pocket screw to sides |
| Fixed bottom | 1 | 11-1/4 x 28-1/2 | Poplar 3/4 | Same |
| Adjustable shelf | 3 | 11 x 28-1/4 | Poplar 3/4 | Ease front edge |
| Face stile | 2 | 2-1/2 x 64 | Poplar 3/4 | Glue and pin |
| Face rail | 2 | 2-1/2 x 24 | Poplar 3/4 | Top and bottom |
| Back panel | 1 | 29 x 64 | 1/4 ply | Brad nail into rabbet |

# Sequence

Mill and cut all parts, label with blue tape, dry-fit the carcass.

Drill shelf-pin rows on both side panels using the jig, check alignment.

Pocket-screw fixed top and bottom to the sides, check for square on the diagonal.

Glue and pin the face frame, clamp overnight.

Cut the back to fit the rabbet, brad nail, then sand to 220.

Two coats wipe-on poly, light buff between coats with a worn 220 sheet.

Frank has the longer clamps if the face frame needs more than four.